Titration Procedure

Portable Biodiesel Unit

- Using the 3 ml syringe, add 1 ml of canola oil to the plastic beaker

- Using the 12 ml syringe, add 10 ml of Solution B

- Add 3 drops of Solution C

- Mix the solutions together by swirling the beaker in a circular fashion (don’t spill or splash any on your hand)

- Using a second 12 ml syringe, draw up 7-10 ml of Solution A

- Drop by drop, add Solution A to the mixture, mixing as you go.

- After approximately 3-4 ml, you will notice the mixture turning pink/magenta.  Continue adding drops until the color stays for at least 10 seconds or more

- Record the amount of Solution A used below

Amount of Solution A used: __________ ml

Plug this number in the spreadsheet.  We will be making 40 gal of biodiesel.  Potassium Hydroxide is 90% pure.

How many grams of KOH are needed to make a 30 gal batch? __________ grams

For more information contact:

Dennis Pennington

Sr. Bioenergy Educator

pennin34@msu.edu

269-838-8265